Product Specification

Mini Cutter

- The tool is a designed hydraulically manually activated tool with two equal, opposite blades.  The top blade remains stationary while the bottom blade comes up when activated for the purpose of cutting small objects.

- The body of the tool should be of anti-corrosive aluminum alloy forging for its lightweight, strength and long life.  The cutter head is swivel capable for easy positioning.

- The maximum cutter opening is 1.5 inches.  The maximum width and depth of cut is 1.5 inches.

- The tool will produce up to 17,000 pounds of cutting force.

- The tool is to be connected to a hand operated pump by a 6 ft. high pressure hydraulic hose with rubber jacket assembly.

- The tool must use genuine Hurst Blue® hydraulic fluid being a patented polyether polyol chemistry, providing additives which provide rust oxidation protection and superior anti-wear properties.  Hurst Blue® is FMRC Approved as a less hazardous hydraulic fluid, 100% synthetic, readily biodegradable, fire resistant and electrically non-conductive, non-corrosive operating medium.

- The tool dimensions shall not be any longer than 8.5 inches, wider than 2.0 inches or higher than 3.0 inches.

- The tool shall be able to tolerate an ambient temperature range of -4F (-19C) up to +161F (+72C).

- The maximum operating pressure to the tool will be 10,000 psi

- The tool must not weigh more than 6.13 lbs.